Source code for generating dtmf signals

close all, clc, clear

%% INITIALIZE VARIABLES

symbol = {'1','2','3','4','5','6','7','8','9','*','0','#'};

lowFreqGroup = [697, 770, 852, 941]; %frequencies in rows

highFreqGroup = [1209, 1336, 1477]; %frequencies in columns

freqArray = [];

for i = 1:4

for j = 1:3

freqArray = [freqArray [lowFreqGroup(i); highFreqGroup(j)]]; %create pairs

end

end

Fs = 8e3; %sampling frequency

N = 8e2; %tones of 100ms

t = (0:N-1)/Fs; %time

sine = 2*pi*t; %sine formula

toneArray = zeros(N, size(freqArray,2)); %array of N*12 for graph

%% VISUALIZE AND SOUND DIGIT SYMBOLS

for i = 1:12 %loop for each of 12 symbols

toneArray(:,i) = sum(sin(freqArray(:,i)*sine)); %make it sinusoidal

subplot(4,3,i);

plot(t*1000,toneArray(:,i)); %make time 100ms

title(['DTMF Signal for "', symbol{i},'"']);

ylabel('Amplitude');

xlabel('Time (ms)');

sound(toneArray(:,i)) %make it sound

pause(0.5)

end

# Source code for dial_number

close all, clc, clear

%% INITIALIZE VARIABLES

symbol = {'1','2','3','4','5','6','7','8','9','*','0','#'};

lowFreqGroup = [697, 770, 852, 941]; %frequencies in rows

highFreqGroup = [1209, 1336, 1477]; %frequencies in columns

freqArray = [];

for i = 1:4

for j = 1:3

freqArray = [freqArray [lowFreqGroup(i); highFreqGroup(j)]]; %create pairs

end

end

Fs = 8e3; %sampling frequency

N = 8e2; %tones of 100ms

t = (0:N-1)/Fs; %time

sine = 2*pi*t; %sine formula

toneArray = zeros(N, size(freqArray,2)); %array of N*12 for graph

digits = '123456789*0#'; %digits to be used

inputNumber = input('Dial a Phone Number: ','s'); %get input

number = [];

silence = zeros(1600,1); %space between digits

%% VISUALIZE NUMBER PRESSED IN DIAL KEYPAD

for i = 1:numel(inputNumber) %loop for each of 12 symbols

pressed = strfind(digits,inputNumber(i)) %find digit pressed

toneArray(:,pressed) = sum(sin(freqArray(:,pressed)*sine)) %make it sinusoidal

figure(1)

sgtitle('Dial KeyPad')

subplot(4,3,pressed)

plot(t*1000,toneArray(:,pressed)) %make time 100ms

title([inputNumber(i), ' Pressed']);

ylabel('Amplitude');

xlabel('Time (ms)');

sound(toneArray(:,pressed)) %make it sound

pause(0.5)

number = [number; toneArray(:,pressed); silence]; %create number array with spaces between digits

end

n = [];

for j=1:length(number)

n(1,j) = j; %create time array

end

figure(2)

sgtitle(inputNumber)

plot(n/8,number)

ylabel('Amplitude');

xlabel('Time (ms)');

Source code of Correlation method of DTMF signaling.

close all, clc, clear

%% INITIALIZE VARIABLES

symbol = {'1','2','3','4','5','6','7','8','9','*','0','#'};

lowFreqGroup = [697, 770, 852, 941]; %frequencies in rows

highFreqGroup = [1209, 1336, 1477]; %frequencies in columns

freqArray = [];

for i = 1:4

for j = 1:3

freqArray = [freqArray [lowFreqGroup(i); highFreqGroup(j)]]; %create pairs

end

end

Fs = 8e3; %sampling frequency

N = 8e2; %tones of 100ms

t = (0:N-1)/Fs; %time

sine = 2*pi*t; %sine formula

toneArray = zeros(N, size(freqArray,2)); %array of N*12 for graph

digits = '123456789*0#'; %digits to be used

inputNumber = input('Dial a Phone Number: ','s'); %get input

number = [];

silence = zeros(1600,1);

%% VISUALIZE NUMBER PRESSED

for i = 1:numel(inputNumber) %loop for each of 12 symbols

pressed = strfind(digits,inputNumber(i)) %find digit pressed

toneArray(:,pressed) = sum(sin(freqArray(:,pressed)*sine)) %make it sinusoidal

number = [number; toneArray(:,pressed); silence]; %create graph of number with silence between digits

end

n = [];

for j=1:length(number)

n(1,j) = j;

end

figure(1)

sgtitle(inputNumber)

plot(n/8,number)

ylabel('Amplitude');

xlabel('Time (ms)');

%% DISPLAY CORRELATION BETWEEN NUMBER AND EACH DIGIT

for i=1:4

digit = sum(sin(freqArray(:,i)*sine)); %take digit signal

[correlation, lag] = xcorr(number, digit); %compute signal similarities

correlationNormalized = correlation./max(abs(correlation(:))); %normalize

figure(2)

subplot(2,2,i)

plot(abs(lag/Fs*900),correlationNormalized,'k')

title(['Correlation Between Number Dialled and Digit of " ',symbol{i},' "']);

ylabel('Normalized Amplitude');

xlabel('Time (ms)');

end

for i=1:4

digit = sum(sin(freqArray(:,(i+4))*sine)); %take digit signal

[correlation, lag] = xcorr(number, digit); %compute signal similarities

correlationNormalized = correlation./max(abs(correlation(:))); %normalize

figure(3)

subplot(2,2,i)

plot(abs(lag/Fs*900),correlationNormalized,'k')

title(['Correlation Between Number Dialled and Digit of " ',symbol{i+4},' "']);

ylabel('Normalized Amplitude');

xlabel('Time (ms)');

end

for i=1:4

digit = sum(sin(freqArray(:,(i+8))*sine)); %take digit signal

[correlation, lag] = xcorr(number, digit); %compute signal similarities

correlationNormalized = correlation./max(abs(correlation(:))); %normalize

figure(4)

subplot(2,2,i)

plot(abs(lag/Fs*900),correlationNormalized,'k')

title(['Correlation Between Number Dialled and Digit of " ',symbol{i+8},' "']);

ylabel('Normalized Amplitude');

xlabel('Time (ms)');

End